# New Robot Configuration

| Model | CQR-1500 | SN | CQR152401080057 |
| --- | --- | --- | --- |
| Engineer | Allen Cheung | Date | Jan 8 2024 |

## Network Topology

| Local LAN | SSID | Laser_Cart |  |
| --- | --- | --- | --- |
|  | PW | okagv2015 |  |
|  | IP | 192.168.1.1 |  |
| CQR-1500 | MOXA-1137C | WLAN | 192.168.1.157 |
|  |  |  | ID:admin |
|  |  |  | PW:moxa |
|  |  | LAN | 192.168.0.1 |
|  | RASP-PI | WLAN | 192.168.1.57 |
|  |  | ETH0 | 192.168.0.3 |
|  |  | ETH1 | 192.168.126.252 |
|  | LRC-600 | ETH | 192.168.0.2 |
|  |  | Remote server | 192.168.1.2/9007 |
|  |  |  | 192.168.1.251/51888 |
|  |  |  | 192.168.1.252/51888 |
|  |  |  | 192.168.0.3/51888 |
|  |  |  | 192.168.126.253/51888(wired) |

## Network settings on the computer (Already configured on the Y4B computer)

There are two network settings on the computer:

To get to the Internet Protocol Version 4 (TCP/IPv4) Properties, follow the steps below:

Control Panel > Network and Internet > Network and Sharing Center > Connections > Properties > Internet Protocol Version 4 (TCP/IPv4) > Properties

- Wifi to Laser_Cart:

  - Static IP:

- Ethernet Cable directly connected to the robot:

  - Static IP: 192.168.126.253

## Run VNC Viewer

While the ethernet cable is being connected, run the VNC Viewer program.

- Connecting to the Raspberry Pie IP 192.168.126.252 with ID: liftians and password being the same as of MM. The 192.168.126.252 IP is the static IP within the Raspberry Pie environment

- Inside the VNC Viewer, open the web browser to access 192.168.0.1 Moxa page to change the password and other network settings

- Save and restart the Moxa program to make the changes effective. It will take about one minute to restart the Moxa router

## Log in to Moxa page outside VNC Viewer

- Open the web browser with IP: 192.168.1.1XX, XX is the robot number. Click on Import/Export on the home page.

- Import the network configuration file from bot 51.

Choose File (Desktop > NewRobotSetup > Config_51 ) > Open > Import Configuration

- Change the IP address to 192.168.1.1XX from 192.168.1.151

- General Setup > Network Settings

- Save Configuration and Restart

## Configuration

Use the network configuration for Bot 51 as HWF network connections (Primary and Secondary Servers)

RoboDemo connection > RoboCheck > Protocal Switch to LiftiansTest > RoboConfig to select an IP > Read from Network Param> Modify Network parameters and Write

| Network Config (HWF) | Robo. Model Config. |
| --- | --- |
| Robo. Control Config. | Robo. Info. |